Laser Technical brief #1

Laser starts  Up and Monitor is not working

- When system starts up and the monitor is not connected, the unit defaults to internal monitor

- To get monitor back, Wait until unit is powered up (30 sec) then hit <Cntrl><ALT><F1> which should reset monitor settings

- We are aware the Windows Embed system needs Monitor, keyboard and mouse.

The bios password is "embedded".

It is supposed to be setup here with no keyboard and mouse required, but we must have a glitch somewhere in the process.  Bob has alerted the necessary parties here

Laser Technical brief #2

Merlin will not start and give s error- cannot open empty folder

- The system is trying to start and the software will not open- this typically happens when a virtual port is added by another software  (USB RFID driver)

- Merlin is trying to start and it cannot because the virtual port is no longer there.

- Go to- C:\telesis\laser\ Merlin\params and delete the HOST file in that directory.